[2016 cvpr A 3D morphable model learned from 10000 faces]

Contributions:

- Introduced a novel automated and robust Morphable Model construction pipeline.

- The dataset includes rich demographic information about each subject(age, gender, ethnicity groups, and proved to have  quality and descriptive power

- Qualitative and quantitative evaluation.

- Demonstrate that large-scale model with accurate demographics enables accurate age classification from 3D shape data alone.

- General idea:

- Construction of 3DMM consists two steps:

- dense correspondence (training set of facial meshes)

- and performing statistical analysis on registered data to produce low-dimensional model

- Dense correspondence: ‘UV’ map(flattening 3D surface down into a 2D space)

- Similarity alignment & statistical modeling: PCA generating a 3D deformable model as a linear basis of shapes.

- M∈R3n is the mean shape（平均形状）

- U=[U1,U2,...Ud]∈R3nxd, orthonormal basis matrix, each column is eigenvectors Ui, (酉矩阵的前d列)

- α=[α1,...,αd]∈Rd is the shape vector that contains the parameters (coefficients) that define a specific shape instance (特定的实例所具有的参数)

- Input 3D mesh X, X can be projected on the model subspace by finding the shape vector α, α generate a shape instance(1), as close as possible to X.

- Corresponding projection P(X) on subspace are given by:

- [62] L. Tran and X. Liu. Nonlinear 3d face morphable model. In In Proceeding of IEEE Computer Vision and Pattern Recognition, Salt Lake City, UT, June 2018. 1, 3, 7, 8, 9

- [63] L. Tran and X. Liu. On learning 3d face morphable model from in-the-wild images. arXiv:1808.09560, 2018. 1, 2, 3

- [59] A. Tewari, M. Zollhofer, P. Garrido, F. Bernard, H. Kim, ¨ P. Perez, and C. Theobalt. Self-supervised multi-level face ‘model learning for monocular reconstruction at over 250 hz.In The IEEE Conference on Computer Vision and Pattern Recognition (CVPR), 2018. 1, 2, 3, 6, 7, 8

- FML: Face Model Learning from Videos

- Disentangle facial shape, appearance, expression, scene illumination

- Flaws of Previous method:

- Data-driven priors required 3D face scans

- Depth ambiguity

- Optimization-based reconstruction rely on a personal model or parametric prior, require ground truth face geometry

- Another Fully unsupervised training see in:

- (2017 ICCV)MoFA: Model-based Deep Convolutional Face Autoencoder for Unsupervised Monocular Reconstruction

- Decoder get input of detailed code vector with face pose, shape, expression, skin reflectance and scene illumination encoded from a CNN-based encoder.

Net work architecture (unsupervised)

Acquire code vector: CNN network

Reconstruct 3D dense correspondence from code vector: Generative model

CNN Encoder-->model-based decoder + parameterize pose/shape/expression/skin reflectance/illumination-->realistic synthetic image ----> compared with input image --->so it is unsupervised end-to-end model

(2018 arXiv)On learning 3D Face Morphable Model from In-the-wild Images

Contribution: Learn a nonlinear 3DMM model from a large set of in-the-wild face images, without collecting 3D face scans.

Pipeline: Image->feed to network encoder->estimate parameters (projection, lighting, shape and albedo parameters).

Nonlinear 3DMM-like model: 2 decoders

- Decoder1: shape -> 3D shape

- Decoder2: albedo parameters->albedo

Designed a analytically-differentiable rendering layer to reconstruct the original input faces use parameter(projection parameter, lighting, 3D shape, albedo).

- Contribution: training a multi-frame video-based self-supervised deep network that:

- Learn a face identity model

- Jointly learning to reconstruct 3D faces

- By introduce (1)multiframe consistency loss (consistent shape and appearance across multiple frames of a subject’s face).

Face Identity Model(self-supervised)

Component 2

Component 1

Appearance

(in terms of albedo)

Geometry

(modulo expression)

- 引用表达：

- Monocular image-based 3D reconstruction of faces is a long-standing problem in computer vision.

- In order to tackle the difficult monocular 3D face reconstruction problem, most existing methods rely on the availability of strong prior models that serve as regularizers for an otherwise ill-posed problem[]

- The same results/limitations: xxx and xxx share the same results/limitations.